ADOPTED BY THE CITY OF MILTON MAYOR & CITY COUNCIL ON

DECEMBER 21, 2006

AMENDED JUNE 16, 2008

ARTICLE XII-H(2)

Birmingham Crossroads of the Northwest Fulton Overlay District

12H(2).1. 	PURPOSE AND INTENT. The Mayor and Council of the City of Milton, Georgia

hereby declares the purpose and intent of this ordinance to be as follows:

To implement the Birmingham Crossroads Plan, the Birmingham Design Guidelines

and the 2015 North Fulton Comprehensive Plan “Maintaining the Rural Character in

Northwest Fulton County.

To implement village type pattern development at the Birmingham Crossroads by

having buildings with a pedestrian scale, variation in building size, architectural

detailing, variation in building massing, and street-orientation.

To protect at least 10% of the Birmingham Crossroads as open space. To have a village green incorporated in the development of the Northeast and Southeast quadrants of the Crossroads that gives the continuous appearance on both sides of Birmingham Road.

To promote a pedestrian oriented development by dividing the land in the Birmingham

Crossroads into small walkable blocks with the construction of an internal road system.

To balance the needs of pedestrians and automobiles by incorporating on-street parking, cross walks, pedestrian crossings, landscape strips, alternative paths and sidewalks along existing and internal roads.

To contain development at the Birmingham Crossroads, within the physical boundaries of 27.1 acres, by placing septic systems at the perimeter of non residential development and then by having a buffer at the exterior of the septic systems.

To encourage the preservation of historic resources and to encourage incorporation of

historic resources identified in the 1996 North Fulton Historic Resources Survey into

new developments.

To encourage the preservation of the rural area by preserving the night sky.

12H(2).2. 	BIRMINGHAM CROSSROADS REGULATIONS. The Birmingham Crossroads is

a section of the Northwest Fulton Overlay District. This article, XII-H(2), applies to all parcels included in the Birmingham Crossroads Plan of March 3, 2004, excluding single residential uses.

The Birmingham Crossroads is located at the intersection of Birmingham Highway,

Birmingham Road and Hickory Flat Road. The boundary of the Birmingham

Crossroads section of the Northwest Fulton Overlay (i.e. this article XII-H(2)) shall

include all parcels within the outlined area on the map below. The Birmingham

Crossroads section of the Northwest Fulton Overlay is 27.1 acres and contains the historic mixed-use center of Birmingham and land surrounding it.

If standards are not specified in this Article, then standards of the Northwest Fulton

Overlay District (Article XII-H) shall apply. If standards are not specified in the

Northwest Fulton Overlay District, then the City of Milton Zoning Ordinance shall

apply.

Whenever provisions of this Article conflict with any other Article in the Zoning

Resolution of Fulton County or any other Fulton County ordinances, regulations, or

resolutions, these standards shall prevail.

12H(2).3. 	DESIGN REVIEW BOARD.

The City of Milton Design Review Board (Article XII-H) as set

forth in Section 12H.2 of the ordinance, shall review for approval all plans for development in Birmingham Crossroads for compliance with the standards herein and “the Birmingham Crossroads Guidelines”  prior to the approval of a Land Disturbance

Permit, Building Permit and Demolition Permits for both residential and non-residential structures. The Design Review Board shall only make recommendations to the Board of Zoning Appeals for Primary Variances.

12H(2).4. 	DEVELOPMENT STANDARDS.

A. 	Streets

Notwithstanding the provisions of the Fulton County Subdivision Regulations and

requirements of the City of Milton Transportation Engineer, existing and new streets

within the Birmingham Crossroads shall conform to the design standards and location

criteria set forth below. When these standards conflict with the Georgia State

Department of Transportation, the State’s standards may prevail.

The Birmingham Plan calls for the existing roads – Birmingham Highway, Birmingham Road and Hickory Flat Road to promote village and pedestrian oriented development by balancing the needs of pedestrians and automobiles.

1. 	Existing Roads- The existing roads, Birmingham Highway, Birmingham Road,

Hickory Flat Road, shall meet the following standards

a. 	Minimum Landscape Strip: 6 feet

b. 	Minimum Sidewalk width: 6 feet

c.	Building setback: 10 feet maximum in the Village center and 20 feet

maximum in the Rural Section.

d. 	On street parallel parking: 8 ft wide on county roads and bulb-out at the

intersections when possible on County roads

Internal Road Cross section

2. 	Internal Road System- Each quadrant shall have an internal road that will link

two existing roads. Curb cuts from existing roads shall be minimal. Instead

access to each quadrant shall be from an internal road. Internal roads shall meet the following standards:

a. Travel lanes: 10 feet

b. Minimum Landscape Strip: 6 feet

c. Minimum Sidewalk width: 5 feet

d. Building setback: 0 to 10 feet maximum in the Village center and 20 feet

maximum in the Rural Section.

e. On street parallel parking: 7 ft wide

B. 	Landscaping

1. 	Specimen trees should be preserved to the extent possible. The

landscaping shall reflect the rural context. It should be simple, informal,

naturalistic in design and use native and naturalized vegetation.

2. 	Shade trees in the landscape strips shall be planted in asymmetrical

groupings at a minimum density of one tree per 30 feet of street frontage.

Trees shall be selected from the City of Milton Tree Preservation

Ordinance and Administrative Guidelines. The Georgia Department of

Transportation may have some limitation on the planting of street trees

along Birmingham Hwy.

3. 	Street trees may be counted towards the required tree density for a site as

approved by the City of Milton Arborist.

4. 	Street trees shall be trimmed up 7 feet to not impede pedestrians.

5. 	Street lights and pedestrian lights may be placed in the landscape strip.

6. 	Landscape islands in parking lots shall meet standards in section 4.23. of

the Zoning Ordinance.

7. 	Street trees shall be a minimum of 4” caliper or as approved by the

City of Milton Arborist.

C. 	Sidewalks and Pedestrian Paths

1. 	Sidewalks or pedestrian paths are required along all public and private

road frontages and may meander around existing trees subject to the

approval of the Fulton County Arborist.

2. 	Pedestrian paths may be installed instead of sidewalks as approved by

the Manager of Community Development Department.

3. 	Sidewalks widths shall be a specified in section 12H(2).4. Pedestrian

paths shall be a minimum of 5 feet wide. They shall be made out of a

hard surface material such as concrete, brick or pavers. Paths may be

gravel or gravel dust as approved by the Community Development Manager.

4. 	Sidewalks for all new projects should connect with existing walks, where

applicable.

5. 	Paths from the sidewalk to the buildings, between buildings, and to

parking lots should be established with minimal interruption by vehicular

circulation, parking lots, and service areas.

6. 	Pedestrian paths shall be designed to minimize automobile and

pedestrian interaction.

7. 	To the extent feasible, paths shall be designed to connect to existing or

future paths and to developments adjacent to the Crossroads.

8. 	Sidewalks and pedestrian paths shall be indicated on the site plan

submitted at the time of the application for a Land Disturbance Permit.

D. 	Street Furniture

1. 	The Birmingham Crossroads shall have consistent streetscape furniture,

including, but not limited to, benches, trash baskets, newspaper

dispensers.

2. 	If provided, street furniture shall be located outside of the minimum

sidewalk width of 5 ft.

3. 	The design of the streetscape furniture shall be timber form construction

as suggested in the Design Guidelines and is subject to approval of the

Manager based on a recommendation of the NW Fulton DRB.

4. 	Hardscape elements such as wider sidewalks plazas, street furniture such

as benches, fountains, tables and chairs, and trash receptacles may be

located in the required front yard and side yard setbacks.

E. 	Streetscape Lighting

1. 	A single style and color of light fixtures and poles should be used

throughout the Crossroads development. It shall be compatible with the

streetscape furniture.

2. 	The design of the streetscape lighting should have a galvanized metal

hood, with black pole, arm and brace as suggested in the Design

Guidelines. Lighting fixtures are subject to approval of the Manager

based on a recommendation of the NW Fulton DRB and per the approval

of the City of Milton Community Services Department.

3. 	All lighting should be fitted with cut-fixtures to prevent light spillover.

4. 	Pedestrian lighting shall be a maximum of 15 feet high. They should be

installed at 40 to 60 feet intervals along sidewalks with a maximum 2

foot candle power.

5. 	The lighting plan for sidewalks shall be included on the site plan

submitted at the time of application for a Land Disturbance Permit.

6. 	All exterior lighting fixtures (luminaires) shall be either full cutoff or

cutoff type.

7. 	Light trespass (spill light) at a residential or nature preserve property line

shall not exceed 0.1 fc vertical at 3 feet above grade. Light trespass at

other property lines shall not exceed 0.5 fc vertical at 3 feet above grade.

8. 	Light levels in all areas may not exceed the recommended limits or

values established by the IESNA Handbook, 8th edition.

9. 	Externally illuminated signs must be illuminated from the top- shining

downwards, not exceeding 15 fc.

10. 	Up lighting of flags, steeples, monuments and buildings must use narrow

beam, shielded luminaries not to exceed 7 fc.

11.	 For drive-under canopies and pump islands, the average horizontal

luminance on the pavement shall not exceed 7 fc under the canopy. The

luminaire shall be recessed into the canopy ceiling so that the bottom of

the luminaire does not extend below the ceiling.

12. 	Full cutoff luminaries only shall be used for all service and security

lighting.

13. 	Sodium vapor luminaries (yellow light), promotional beacons, search

lights, laser source lights, strobe lights or any similar light, out-of-season

lighting (for example Christmas lights), series, lines or rows of lights,

flashing, changeable message or reader boards and lighting used for

causing sky glow to attract attention are prohibited.

14. 	Lighting allowed under state and federal law, outdoor luminaries existing

and legally installed prior to the effective date of this amendment,

emergency lighting by police, fire, ambulance and rescue, and light

sources for holiday decorations (but not outdoor seasonal sales) are

exempt from the provisions of this ordinance.

F. Screening

1. 	To the extent possible, items shall be screened by placement to the rear

of a building.

2. 	Dumpsters shall be screened from view from adjacent roads, sidewalks

and paths. In addition, the dumpster shall be enclosed on three sides with

a masonry wall. The wall should be faced with brick or natural stone and

compatible with the adjacent architectural design, materials and colors.

The enclosure shall be a foot higher than what is contained in the

interior. To the extent possible, a common location for dumpsters should

be used.

3. 	Loading and other service areas shall be located to the rear of buildings

where least visible. Evergreen trees should be used to screen views of

service areas.

4. 	Parking lots, bumpers, wheels and paving shall be screened with

vegetation planted between 30-36 inches tall. Evergreen shrubs should

be used for low-level screening of parking lots, sidewalks. Berms

without landscaping are not an appropriate screening technique.

5. 	Rooftop and building mounted mechanical and electrical equipment shall

be screened from view by a parapet wall or roof screen. If on the ground,

equipment shall be screened with landscaping.

G. 	Fencing

1. 	Allowed fencing types and materials are: split rail, stacked stone wall or

brick columns with horizontal boards, picket (not off the shelf variety),

vegetative hedges, 3-4 horizontal board horizontal rail, horse wire ( no

climb 2” x 4”) and no climb wire (see examples and photos below).

Posts shall be no more than 6” taller than fence.

2. 	Horse wire (no climb 2” x 4”) and chain link fence around retention

areas must be clad in black vinyl.

3. 	Prohibited fencing materials are: unclad chain link, vinyl, PVC,

aluminum picket, and metal.

H. 	Parking Lots

- Parking lots shall be located behind and to the side of a building.

2. 	The amount of parking required by the City of Milton Zoning Ordinance

shall be reduced by 10%.

3. 	Parking lots shall be divided into small contained areas through the use

of perimeter landscaping and canopy trees. Parking lot landscaping shall

follow the standards of the Tree Ordinance.

4. 	Parking lots shall be surfaced with crushed gravel on appropriate base,

asphalt, concrete, or exposed aggregate. Pervious materials such as

gravel and grasscrete pavers (not bituminous paving) could be used and

particularly in areas that have parking spaces in excess of the minimum

amount required by the City of Milton.

5. 	Marked on-street parking shall be counted as part of the minimum

number of required parking spaces.

I. 	Open Space

1. 	A minimum of 10% of open space shall be set aside in each overall

development. This shall not include septic fields, detention facilities or

landscape islands. To the extent possible, the open space should be

clustered as one single open space.

2. 	Within land designated as Retail Services in the 2015 Land Use Map in

the Northeast and Southeast quadrants, land shall be set aside as a

“Village Green”. The size of the Village Green in each quadrant shall be

as set forth in the table below.

3. 	The Village Green shall be a simple lawn area canopy trees serviced by

sidewalks. There could be hardscape features such as plazas, benches,

lighting and other furniture to serve pedestrians. The park should be

designed to be an attractive day and evening setting for either individual usage or special community events.

J. 	Miscellaneous Provisions

1. 	Except as provided for in Article 19, the storage and/or sale of goods is

prohibited in parking lots and other areas outside of the interior or

permanently sheltered portions of a building.

2.	 LP tanks and ice machines are allowed outside provided they are

screened from view.

3. 	Storage of shopping carts is allowed on the side of buildings without a

permit.

4. 	Shopping carts shall be screened.

5. 	Hours of operations shall be from 6:30 a.m. to 11:00 p.m.

6.	Free-standing fast food restaurants are prohibited. (Amended 10/06/04)

7. 	Except for one (1) lane for a drug store and two (2) lanes for a financial

institution, drive-thrus are prohibited. Drive-thrus shall be screened

from view from the public right-of-way subject to the approval of the

Manager based on a recommendation by the Northwest Fulton Design

Review Board and the City of Milton Arborist. (Added 10/06/04)

8.	Drive-ins/ups are prohibited. (Added 10/06/04)

12H(2).5. 	BUILDING SETBACK AND DESIGN.

All nonresidential standards apply to existing and new structures, used for nonresidential purposes, within the area defined as the Birmingham Crossroads.

Buildings should be designed in conformance with traditional historic building

placement and massing. Buildings may be executed using historic styles, or may be

new and innovative interpretations of local architectural traditions. Details must be

simple and concise, erring on the side of austerity. Avoid nostalgia and excess

ornamentation, and strive for direct and bold expression.

Building designs area categorized as “Village center” or “Rural section” depending on

their location within the Birmingham Crossroads. Village center buildings are to be

located along the existing roads and within 400 feet of the intersection of Birmingham

Highway, Birmingham Road and Hickory Flat Road. Rural section buildings are to be

located beyond 400 feet of this intersection. The location of both is indicated in the map below. Village center buildings shall be placed at the property frontage. The most

prominent aspect design aspect is the street façade. Rural buildings are placed away

from the core line forming a more casual transition to the “Countryside”.

A. 	Building setback

1. 	Village center buildings shall be set back 10 feet maximum from existing

roads and/or internal roads and drives. (Amended 11/03/04).

2. 	Rural section buildings shall be set back 20 feet maximum from the

property line.

3. 	Village core buildings shall be located at least 75 feet from the overall

development boundaries.

B. 	Building Design

1. 	Buildings at the Village center shall have village/main street

characteristics (as shown in drawings a & b below). Buildings in the

rural section of the crossroads should have an informal rural design (as

shown in drawings d & c).

2. 	Buildings shall have a variety in their size, massing, height (including

two-story buildings) and detailing. Visual interest shall be achieved

through the use of at least one of the following architectural element:

balcony, porch, marquee, or façade off-sets.

3. 	Buildings shall be oriented to sidewalk and street.

4. 	Primary building entrances shall face the sidewalk and street. Secondary

entrances may be located on side and rear elevations.

5. 	Building facades facing a right of way shall have buildings wall offsets,

including projections and recesses, every 40 feet to provide architectural

and visual interest and variety.

6. 	Trim, fenestration, materials and composition shall be present on all

sides. The facades of buildings in the Village Core may be more

pronounced by the incorporation of architectural elements than other

elevations.

7. 	The building materials applied to the front façade shall continue in the

same proportion on all exterior elevations.

8. 	Allowed exterior wall materials are: horizontal lap siding, board and

batten, wood shingle, wood mould brick, and stone. Metal shingle,

corrugated or 5-v metal are allowed for agricultural style buildings in the

rural section. Avoid novelty shapes on shingles. Clapboard must be

wood or cementitious. Board and batten must be wood or cementitious

9. 	Prohibited exterior materials are: synthetic stone, concrete stone veneer,

stucco and exterior insulation finish material, vinyl and metal siding.

Prohibited trim materials are: Howe casings, manufactured dentils,

standard brick mould.

10. 	Allowed foundation materials are stacked stone, cut stone and brick.

Rural foundation materials can be poured concrete finished with smooth

stucco, brick, or stacked stone. Foundations may be formed out of

poured concrete or concrete masonry units.

11. 	Porches shall have stone, brick or masonry piers. Concrete masonry units

shall be finished in smooth stucco on masonry or concrete formed of

horizontal wooden boards.

12. 	Windows:

a. 	Windows that open to the interior shall be included on elevations

facing a sidewalk or street. Windows in the front façade shall be

predominantly vertical in orientation.

b. 	Windows shall be present on side elevations. Village core

buildings can have minimal windows on the sides and rear of the

building.

c. 	For Village Core Buildings windows or glass storefronts are

required on elevations facing a street. Storefront windows (large

windows located on the façade with the main entrance) shall

comprise a maximum of 50% of the façade width.

d. 	The size of any individual plate glass panel for store front

windows is limited to 32 square feet. Panels may be grouped

together with heavy mulls separating the panels. Glass

storefronts must be a minimum 9’0” in height.

e. 	Storefronts on the street frontage may be built with entryways

recessed from the sidewalk, but not exceeding 50% of the façade

width.

f. 	Facades with a storefront shall have a contrasting masonry base

1-6” to 2’ feet in height.

g. 	Windows not located on the façade must be true divided lite (real

muntins, or TDL), simulated divided lite (glass sandwiched

between glass, or single divided lights-SDL). They may be

wood, clad wood, or polymer and must be paint grade. Industrial

metal sash is acceptable (not to be used on residential doublehung

type windows).

h. 	Tinted and reflective glass, GBGs (grill between glass), reflective

glazing, and pop in grilles are prohibited.

13. 	Shutters may be solid core polymers, or durable hardwoods. Vinyl, nailon,

false wood graining and pre-finished shutters are prohibited.

14. 	Village Core doors can be wood or a combination of wood and glass.

15. 	Rural section doors shall be wood.

16. 	Garage doors must be utilitarian in character and may be wood or metal

17. 	Plastic laminated, standard 6 panels stamped metal, and leaded/beveled

glass lights are prohibited. Standard garage paneled doors are prohibited.

Avoid faux strap hinges, and embellishments.

18. 	Chimneys stacks must be faced in masonry material. Sheet goods

including hardboard stucco and siding are prohibited on chimneys.

Metal stacks are acceptable.

19. 	Porches are encouraged throughout the Birmingham Crossroads. Village

Core buildings are encouraged to have a 1 or 2 story porch on the street

frontages or a marquee on the first story. Rural buildings are encouraged

to have front porches that span at least 75% of the façade.

20. 	Porch columns must be wood, resin material, cast iron, brick, stone or

masonry. Allowed columns types are Greek Doric, fluted or plain. Full 2

story and sheet metal columns are prohibited.

21. 	Railing systems shall be painted wood or metal. Railings must be simple,

round, plain or tapered.

22. 	Synthetic and pre-fabricated railing systems, classical balusters and

spindle-work, synthetic composition decking material are prohibited.

23. 	There shall be no pitch requirement for a roof. Village core buildings

must have low pitched roofs or must have flat roofs surrounded by

parapet walls that screen mechanical units. Rural buildings must be

pitched.

24. 	Allows roofing materials are: wood shingles, wood shakes, standing

seam metal, corrugated metal, 5-V paint grip galvanized metal or slate.

Flat roofs may be membranes if not visible from the right-of-way. Prefinished

metal roofs are prohibited.

25. 	Vents and stacks shall be painted and hidden from view to the greatest

extent possible.

C. 	Colors

All buildings shall meet these color standards. Earth tone and off-white colors

are encouraged. All exterior building materials, architectural and decorative

elements and sign structure colors must be selected from the list provided. Brick

should be wood mould in the brown to Georgia red clay range. Mortar colors

range from river sand to native clay. Non-historic brick textures and colors such

as pink brick, salt and pepper effects are prohibited.

The numbers refer to Pantone Matching System, an international color matching

system. All shades of whites and off whites shall be allowed.

| Permitted Colors |  |  |  |
| --- | --- | --- | --- |
| Base Colors - Primary Building material Pantone Colors |  | Accent Colors - allowed for decorative elements, roof, accents, and sign structure Pantone Colors |  |
| 120C 1205C 160C 1815C 2915 400C 406C 404C 410C 413C 420C | 466 C&U 4645U 468C&U 4685C 4715C 482U 483C 490C 5455C 5477C | 1807C 2C-7C 289C 316C 401-405C 407-412C 412C 415-419C 423C 424-425C 448-450C | 4975 553 5363 539 548 5467 5743U 5747U 5757U 5773U 5815U |

| Permitted Colors |  |  |  |
| --- | --- | --- | --- |
| Base Colors - Primary Building material Pantone Colors |  | Accent Colors - allowed for decorative elements, roof, accents, and sign structure Pantone Colors |  |
| 421-422C 435C 4495C 450C 452C 4525C 4535C 4535U 4545C 454C 4545C | 549U 5773C 5787U 5793U 5803U 5807U 5855U 5875U 607 608 623U 726 Warm Grey 1-4 | 4485U 4495C 451C 4505C 4515-4525C 455C 462U 464U 476U 478U 484C 491C | 5835U 625U 627U Warm Grey 5-7C Warm Grey 8-11 |

12H(2).6. EXISTING HISTORIC STRUCTURES (as identified in the 1996 Historic Resources Survey).

- Alterations and additions shall be consistent and reinforce the historic architectural character of the entire structure and shall comply with the standards herein.

- New additions and exterior alterations shall not destroy historic materials that characterize the property. The new work may be differentiated from the old. To protect historic integrity, any new work shall be compatible with the massing, size, scale and architectural features of the property.

- The removal of distinctive materials or alteration of features that characterize a structure shall be avoided.

- Where the severity of deterioration requires replacement of a distinctive feature, the new feature shall match the old in design.